Thank you for purchasing one of our quality Locash Racing products.

Product: 	OBD Conversion Harness

Model:		OBD0 to OBD2a JDM ECU

# Part #:		OBD0-2A-JDM

In order to complete the installation of your conversion harness, additional sensors and/or equipment may need to be connected to the subharness.

Color			Function

Purple 			Vtec Solenoid Signal

Blue 			O2 Sensor Heater Control

Green 			Intake Air Bypass Signal (P72/P13 ECUs only)

White 			O2 Sensor Signal

Orange 			Knock Sensor Signal

# Grey 			Vtec Pressure Switch Signal

Red 			Power -O2 Sensor

Yellow 			Service Check Signal

Pink 			Power– Intake Air Bypass

Black X3 		Ground (3)- Service Check-Vtec Pressure Switch-O2 Sensor

Installation Notes:

IAB-  					Used only with P72 or P13 ECUs

Service Check (ground and signal)-  	These connections have been added for your convenience only, a simple toggle switch can be used to connect the two wires.  The use of this alternate service check connection is optional, if you choose to continue using the original system, simply cap the connections so as to not accidentally ground the signal wire and activate the service check circuit.

Powers and Grounds-	These are suggested connections,  other suitable locations may be used for powers and grounds.

Distributor Wiring Comparison:

OBD0				OBD2

Blue w/Green Stripe		Yellow

Blue w/Yellow Stripe		Black

Orange w/Blue Stripe		Green

White w/Blue Stripe		Red

Orange				Blue

White				White

Thick White			Yellow w/Green Stripe   (Ignitor Signal)

Alternator Wiring Comparison:

OBD0				OBD2

Yellow				White w/Red Stripe

White w/Yellow Stripe		White w/Green Stripe

# Black w/Yellow Stripe		Black w/Yellow Stripe

White w/Blue Stripe		White w/Blue Stripe

If you are using OBD1 or OBD2 injectors, remove the injector resistor box

and connect all five wires that were originally going to the resistor

box together.

If you have any questions, please email info@locashracing.com