Thank you for purchasing one of our quality Locash Racing products.

Product: 	OBD Conversion Harness (Purchased after May, 1st, 2009)

Model:		OBD0 to OBD1 ECU

Part #:		OBD0-1-Black

In order to complete the installation of your conversion harness, additional sensors and/or equipment may need to be connected to the subharness.

Color		Function

# Black		Vtec Solenoid Signal

# Brown		O2 Sensor Heater Control

Red		Intake Air Bypass Signal (P72/P13 ECUs only)

Grey		O2 Sensor Signal

White		Knock Sensor Signal

Green		Vtec Pressure Switch Signal

Blue		Service Check Signal

Purple   	Ground - O2 Sensor

Orange		Ground – Service Check Signal

Yellow		Power – O2 Sensor

Installation Notes:

IAB-  	Only for P72 or P13 ECUs (AKA Secondary Butterflies)

Service Check (ground and signal)-  	These connections have been added for your convenience only, a simple toggle switch can be used to connect the two wires.  The use of this alternate service check connection is optional, if you choose to continue using the original system, simply cap the connections so as to not accidentally ground the signal wire and activate the service check circuit.

IMPORTANT:

-The above connections may not be used in all configurations, JDM ECUs will not require most of the extra sensors.

-Please consult a professional installer in order to avoid serious damage to the vehicle’s components.

This product is designed for off road use only

Any questions or comments, please feel free to contact us at info@locashracing.com

## Additional Information

### OBD0 to OBD1 Distributor Wiring

OBD0 (Harness Side)		OBD1 OEM Dist.(Distributor Side)

Blue w/Green Stripe		Orange*

Blue w/Yellow Stripe		White*

Orange w/Blue Stripe		Orange w/Blue Stripe

White w/Blue Stripe		White w/Blue Stripe

Orange				Blue w/Green Stripe

Thin White			Blue w/Yellow Stripe

Thick White**			Yellow w/Green Stripe

*Note 1: On some aftermarket OBD1 distributors, the Orange and the White wire are swapped.

**Note 2: If you confuse the two OBD0 white wires, the car will not start and could cause ignitor failure!

### Injector Resistor Box Info

If you are using OBD1 fuel injectors, remove the injector resistor box and connect all five wires (that were originally connected to the resistor box) together.

#### Special Instructions if used with Hasport Wiring Conversion

If you are using our ecu adapter with a Hasport OBD1 converted 88-91 harness, you will only need to connect the following wires, all other wires on the subharness will not be used.  Make sure that you connect the Yellow, Orange, Red, and White wires from the Hasport harness to their respective spots on the factory ecu plugs (not the ecu adapter).

Hasport harness 	Locash Racing harness	Sensor/Item

Green w/Yellow Stripe		Black			VTEC Solenoid

White w/Red Stripe		Grey			O2 Sensor Signal

Blue w/Black Stripe		Green			VTEC Oil Pressure

Blue w/Red Stripe		White			Knock Sensor

Orange w/Black Stripe		Brown			O2 Sensor Heater Control

Red w/White Stripe		Red			IAB (GSR and H22A Only)

##### O2 Sensor Wiring Information for various Honda Oxygen Sensors

Ecu Adapter Color	O2 Sensor 	or	O2 Sensor	or	O2 Sensor

Grey			White			White			White

Brown			Orange/Black		Black (next to green)	Orange/White

Yellow			Yellow/Black		Black (next to white)	Yellow/Black

Purple			Green/White		Green			Green/White