Thank you for purchasing one of our quality Locash Racing products.

Product: 	OBD Conversion Harness

Model:		OBD1 to OBD2a ECU

Part #:		OBD1-2a

In order to complete the installation of your conversion harness, additional sensors and/or equipment may need to be connected to the subharness.

Color			Function

Black/White		EVAP Bypass Solenoid Valve

Blue/White		Second O2 Sensor Signal

Black/Green		CKF-P

Brown			EVAP Control Canister Vent

Black/Orange		Fuel Tank Pressure

White/Yellow		CKF-M

White/Orange 		Second O2 Sensor Power

White/Green		Second O2 Sensor Heater Control

Black 			Second O2 Sensor Ground

Installation Notes:

-The oxygen sensor ground and power connections provided are optional, other suitable locations may be used.

IMPORTANT:

-The above connections may not be used in all configurations, JDM ECUs will not require most of the extra sensors.

-Please consult a professional installer in order to avoid serious damage to the vehicles components.

Some applications may need

additional connections to the

stock wiring harness:

This product is designed for off road use only

Any questions or comments, please feel free to contact us at info@locashracing.com