Thank you for purchasing one of our quality Locash Racing products.

Product: 	Honda Engine Race Wiring Harness

Part #:		RCWH-1

In order to complete the installation of your race wiring harness, additional sensors and/or equipment may need to be connected.

Color				Function

# Thick Solid Red			12V Power (50 Amp Fuse Recommended)

# Solid Black			Ground

Orange				Switched 12V power (Ignition +12v, 30 Amp Fuse Recommended)

Blue				Tachometer Signal Output

Installation Notes:

There is a ground eyelet on the harness that must be connected to the thermostat housing on the motor.  Failure to connect this ground will cause undesirable operation!

This harness is designed to be used with an OBD2A Honda Style ECU, or an OBD1 Honda Style ECU if an ECU Adapter is used.

Fuel system, starter wiring, and cooling fan wiring are not included.  You will need to have a dedicated fuel system, cooling  and starter switch for your vehicle.

All necessary relays are already wired into the harness.

Please remember to fuse all power connections, no warranty is implied.  If you turn the harness into a pile of molten wire, you may want to double check all connections before purchasing another.

We also recommend running a ground cable from the transmission to the chassis of the vehicle.

Please contact us if you have ANY questions.  info@locashracing.com