Thank you for purchasing one of our quality Locash Racing products.

Product: 	OBD0-2 Engine Subharness (for use with OBD0-2 JDM ECU Adapter)

Model:		OBD0 to OBD2a JDM ECU

Part #:		SUBHARN0-2aJDM

Color				Function

# Green w/Yellow Stripe		Vtec Solenoid Signal

# Blue w/Red Stripe		Knock Sensor

Blue w/Black Stripe		Vtec Oil Pressure Switch Signal

White w/Red Stripe		O2 Sensor Signal

Orange	w/Black Stripe		O2 Heater Control

Yellow w/Black Stripe		O2 Sensor Power

Green w/Black Stripe		O2 Sensor Ground

Red w/White Stripe		IAB Signal  (Only for GSR or H22A/H23A engines)

Black w/Yellow Stripe		IAB Power  (Only for GSR or H22A/H23A engines)

Locash Racing Engine Harness	Locash Racing ECU Adapter		Sensor/Item

Green w/Yellow Stripe				Purple				VTEC Solenoid

White w/Red Stripe				White				O2 Sensor Signal

Blue w/Black Stripe				Gray				VTEC Oil Pressure

Blue w/Red Stripe				Orange				Knock Sensor

Orange w/Black Stripe				Blue				O2 Sensor Heater Control

Red w/White Stripe				Green				IAB (GSR and H22A)

Yellow w/Black Stripe				Red				O2 Sensor Power

Green w/Black Stripe				Black				O2 Sensor Ground

##### Connect the black wire with the eyelet to the thermostat housing, as it will provide the necessary ground for

the VTEC oil pressure switch!