Thank you for purchasing one of our quality Locash Racing products.

Product: 	OBD Conversion Harness

Model:		OBD0 to OBD1 ECU

Part #:		OBD0-1

In order to complete the installation of your conversion harness, additional sensors and/or equipment may need to be connected to the subharness.

Color		Function

# Purple		Vtec Solenoid Signal

# Blue		O2 Sensor Heater Control

Green		Intake Air Bypass Signal (P72/P13 ECUs only)

White		O2 Sensor Signal

Orange		Knock Sensor Signal

Grey		Vtec Pressure Switch Signal

Red (2)		Power -O2 Sensor & IAB

Yellow		Service Check Signal

Black (3)	Ground - Service Check Signal, Vtec Pressure Switch, O2 Sensor

Installation Notes:

IAB-  					Used only with P72 or P13 ECUs

Service Check (ground and signal)-  	These connections have been added for your convenience only, a simple toggle switch can be used to connect the two wires.  The use of this alternate service check connection is optional, if you choose to continue using the original system, simply cap the connections so as to not accidentally ground the signal wire and activate the service check circuit.

Powers and Grounds-	These are suggested connections,  other suitable locations may be used for powers and grounds.

IMPORTANT:

-The above connections may not be used in all configurations, JDM ECUs will not require most of the extra sensors.

-Please consult a professional installer in order to avoid serious damage to the vehicle’s components.

This product is designed for off road use only

Any questions or comments, please feel free to contact us at info@locashracing.com

## Additional Information

### OBD0 to OBD1 Distributor Wiring

OBD0 (Harness Side)		OBD1 OEM Dist.(Distributor Side)

Blue w/Green Stripe		Orange*

Blue w/Yellow Stripe		White*

Orange w/Blue Stripe		Orange w/Blue Stripe

White w/Blue Stripe		White w/Blue Stripe

Orange				Blue w/Green Stripe

Thin White			Blue w/Yellow Stripe

Thick White**			Yellow w/Green Stripe

*Note 1: On some aftermarket OBD1 distributors, the Orange and the White wire are swapped.

**Note 2: If you confuse the two OBD0 white wires, the car will not start and could cause ignitor failure!

### Injector Resistor Box Info

If you are using OBD1 fuel injectors, remove the injector resistor box and connect all five wires (that were originally connected to the resistor box) together.

#### Special Instructions if used with Hasport Wiring Conversion

If you are using our ecu adapter with a Hasport OBD1 converted 88-91 harness, you will only need to connect the following wires, all other wires on the subharness will not be used.  Make sure that you connect the Yellow, Orange, Red, and White wires from the Hasport harness to their respective spots on the factory ecu plugs (not the ecu adapter).

Hasport harness 	Locash Racing harness	Sensor/Item

Green w/Yellow Stripe		Purple			VTEC Solenoid

White w/Red Stripe		White			O2 Sensor Signal

Blue w/Black Stripe		Gray			VTEC Oil Pressure

Blue w/Red Stripe		Orange			Knock Sensor

Orange w/Black Stripe		Blue			O2 Sensor Heater Control

Red w/White Stripe		Green			IAB (GSR and H22A Only)

##### O2 Sensor Wiring Information for various Honda Oxygen Sensors

Ecu Adapter Color	O2 Sensor 	or	O2 Sensor	or	O2 Sensor

White			White			White			White

Blue			Orange/Black		Black (next to green)	Orange/White

Red			Yellow/Black		Black (next to white)	Yellow/Black

Black			Green/White		Green			Green/White